AK Solar Energy Series

Product Specification

| Product Name: | GPS positioning terminal |
| --- | --- |
| Product Model: | AK-GT07E |
| File version: | V1.0 |
| Version date: | 2025.06.18 |

| version | Update content | Reviser | Revision time |
| --- | --- | --- | --- |
| V1.0 | Function description, specification parameters | Ah Xiang | 2025.06.18 |

catalogue

1. Basic product introduction 	3

1.1 Product Introduction 	3

1.2 Application areas 	3

1.3 Product Image 	3

1.4 Product usage introduction 	4

1.4.1 SIM card installation method 	4

1.4.2 Indicator light description 	4

1.4.3 Work logic 	4

2. Functional Features 	5

3. Product parameters 	6

3.1 Hardware specification parameters 	6

- Basic introduction of the product

1.1 Product Introduction

AK-GT07E is a 4G universal solar-powered positioning terminal meticulously crafted for the positioning of cargo fishing boats. It integrates 4G universal wireless communication technology and GPS/BDS satellite navigation and positioning technology. The terminal adopts an industrial-grade, highly integrated, fully built-in antenna design, with a built-in 3-axis sensor and intelligent power-saving wake-up function. It features an SOS emergency button, waterfall detection, rollover detection, and other functions. Three modes can be set to meet various application scenarios of customers, and users can send commands to switch working modes. With the global positioning service platform, the device location can be queried anytime, anywhere.

1.2 Application areas

Insurance industry, corporate fleet industry, automobile manufacturers/4S stores, electric new energy sector, passenger vehicles, taxis, rental vehicles, fishing boats, etc.

1.3 Product images

Two installation methods: 
1. Strong magnetic adsorption 
2. Screw fixation installation

Water immersion detection contactor

Hall tamper sensor

Photosensitive tamper-proof sensor

1.4 Introduction to product usage

1.4.1 SIM card installation method

The device uses a Micro SIM card. The card slot is of a self-popping type. Place the chip side of the card facing the magnetic side, and insert the card into the slot with the notch facing inward.

1.4.2 Indicator light description

Red light - charging indicator light

| The status of the light | meaning |
| --- | --- |
| Chang Liang | Charging |
| "Not bright" | Fully charged/off |

Yellow light - GSM indicator light

| The status of the light | meaning |
| --- | --- |
| Blink once within 2 seconds | GSM initialization |
| Chang Liang | GSM communication is normal |
| Not lit up | GSM sleep/shutdown |

Blue light - GPS indicator light

| The status of the light | meaning |
| --- | --- |
| Blink once within 2 seconds | In satellite signal search |
| Chang Liang | GPS/BDS has been positioned |
| Not lit up | GPS/BDS dormant |

White light - SOS indicator light

| The status of the light | meaning |
| --- | --- |
| flicker | Trigger SOS |
| not bright | SOS not activated |

1.4.3 Working logic

Normal mode: In motion, the device transmits its location every 60 seconds by default. In a stationary state, it transmits its location every 1800 seconds by default. In a dormant state, the device transmits a heartbeat every 5 minutes to maintain a connection with the server. The device can be woken up to a working state by shaking it.

Power-saving mode: In motion state, the device transmits location data once every 60 seconds by default. In stationary state, it transmits location data once every 1800 seconds by default. In sleep state, the device does not transmit data. Disconnecting from the server saves power. The device can be woken up to working state by shaking it.

Intelligent mode: Set the data upload interval, and the device will operate according to the set interval, working for 2 minutes each time, except for

During working hours, the device is in a dormant state and cannot be woken up by vibration.

- Features

- Supports multiple functions such as vibration alarm, overspeed alarm, and removal alarm;

- Support the electronic fence function, where users can define a virtual electronic fence through the platform/APP. When the device enters or exits the fence, it can be recognized by the platform and an alarm will be issued;

- Solar charging, ultra-long battery life, and the ability to continue working without charging in power-saving mode;

- When an alarm occurs, an alert message or call can be sent to the monitoring number, and the alarm can be uploaded to the platform simultaneously;

- The terminal adopts an industrial-grade high-stability GPRS module from a well-known manufacturer, with a built-in GSM high-sensitivity antenna, supporting TCP/IP data transmission and connecting to servers via domain name/IP address;

- Equipped with a large-capacity storage chip, it supports data storage in offline mode and the supplementary transmission of blind-spot data. When the vehicle is in a place with weak wireless signals or severe interference, the onboard device will temporarily store the vehicle's operational data in FLASH. Once the wireless signal returns to normal, it can transmit these data, ensuring no data is lost.;

- Equipped with a built-in 3-axis accelerometer and integrated with precise acceleration algorithms, it can obtain real-time vehicle posture and other vehicle condition assessments;

- High-sensitivity GPS/BDS dual-satellite positioning module, equipped with an anti-interference ceramic antenna, ensures more stable satellite signal search, and supports AGPS for rapid positioning tracking and synchronous timing;

- Three working modes, adaptable to various scenarios.

- Product Parameters

|  | Function Name | have | none | Project function description |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Electrical Characteristics | power supply method | ● |  | battery-powered |  |  |
|  | charge | ● |  | Solar charging & charging cable 5V/1A |  |  |
|  | Operating voltage range | ● |  | DC 3.4V - 4.5V |  |  |
|  | operating current | ● |  | 4V / Average 50mA |  |  |
|  | quiescent current | ● |  | 4V/less than 1mA |  |  |
|  | Built-in battery capacity | ● |  | 10000 mAh (3.7V polymer battery) |  |  |
| Environmental characteristics | Operating temperature range | ● |  | -20℃ - 75℃ |  |  |
|  | Storage temperature range | ● |  | -30℃ - 80℃ |  |  |
|  | Operating humidity range | ● |  | 10%-85% RH non-condensing |  |  |
| Communication characteristics | SIM card | ● |  | Micro SIM card |  |  |
|  | Communication frequency band | ● |  | LTE-FDD | B1/ 2/ 3/ 4/ 5/ 7/ 8/ 12/ 13/ 17/ 18/ 19/ 20/ 25/ 26/ 28/ 66 |
|  |  |  |  | LTE-TDD | B34/ 38/ 39/ 40/ 41 |
|  |  |  |  | GSM/2G | B2/ 3/ 5/ 8 |
|  | Communication antenna | ● |  | built-in antenna |  |  |
|  | Antenna specifications | ● |  | FPC Antenna |  |  |
| GPS/BD positioning features | Positioning module brand/chip model | ● |  | Zhongke Micro AT6558R |  |  |
|  | positioning method | ● |  | BeiDou + GPS |  |  |
|  | Cold start time | ● |  | Average 32 seconds |  |  |
|  | Hot start time | ● |  | On average, 1 second |  |  |
|  | Tracking sensitivity | ● |  | -162 dBm |  |  |
|  | positioning antenna | ● |  | built-in antenna |  |  |
|  | Antenna Specifications | ● |  | 25mm * 25mm * 4mm |  |  |
|  | GPS frequency band | ● |  | L1: 1575.42±1.023MHz |  |  |
|  | BeiDou frequency band | ● |  | B1: 1561.098±2.046MHz |  |  |
|  | Number of satellite channels | ● |  | 32 |  |  |
|  | positioning accuracy | ● |  | <10m（1σ） |  |  |
|  | Timing accuracy | ● |  | <30ns（1σ） |  |  |
|  | speed measurement accuracy | ● |  | <0.1m/s（1σ） |  |  |
|  | maximum acceleration | ● |  | 4g |  |  |
|  | maximum speed | ● |  | 515m/s |  |  |
|  | maximum height | ● |  | 18000m |  |  |
| external interface | Tamper detection | ● |  | Support magnetic Hall tamper detection |  |  |
|  | Water immersion detection | ● |  | Support water-fall detection points |  |  |
| Specifications | Host dimensions (length, width, height) | ● |  | 138mm * 77mm * 30mm |  |  |
|  | Installation method | ● |  | Magnetic attraction/screw (optional: choose one from two) |  |  |
|  | Shell material | ● |  | ABS plastic |  |  |
|  | IP protection rating | ● |  | IP67 |  |  |
|  | host weight | ● |  | 300 G |  |  |