International car charger GPS

Product Specification

| Product Name: | GPS positioning terminal |
| --- | --- |
| Product Model: | C036 |
| File version: | V1.0 |
| Version date: | 2025.6.18 |

| version | Update content | Reviser | Revision time |
| --- | --- | --- | --- |
| V1.0 | Function description, specification parameters | Ah Xiang | 2025.6.18 |

catalogue

1. 产品基本介绍	3

1.1产品简介	3

1.2应用领域	3

1.3产品图片	3

1.4产品使用介绍	4

1.4.1 SIM卡安装方式	4

1.4.2 指示灯说明	4

1.4.3 工作逻辑	4

2. 功能特点	5

3. 产品参数	6

硬件规格参数	6

- Basic introduction of the product

- 1.1 Product Introduction

C036 is a 4G full-network vehicle charging and positioning terminal meticulously crafted for the Internet of Vehicles. It integrates 4G full-network wireless communication technology and GPS/BDS satellite navigation and positioning technology. The terminal adopts an industrial-grade, highly integrated, fully built-in antenna design, with built-in 3-axis sensors and intelligent power-saving wake-up functionality. It features vibration alarm, overspeed alarm, power failure alarm, and other functions. When paired with a global positioning service platform, the device's location can be queried anytime, anywhere.

## 1.2 Application Fields

Insurance industry, corporate fleet industry, automobile manufacturers/4S stores, individual users, electric new energy sector, passenger vehicles, taxi services, rental vehicles, etc.

## 1.3 Product images

- 1.4 Introduction to product usage

### 1.4.1 SIM card installation method

The device uses a nano SIM card. The card slot is of a self-popping type. Place the chip side of the card facing the USB charging port, and insert the card into the slot with the notch facing inward.

### 1.4.2 Indicator light description

Red light - breathing light

| The status of the light | meaning |
| --- | --- |
| slow flash | Power-on is normal |
| Not lit up | The power supply is not connected |

Yellow light - GSM indicator light

| The status of the light | meaning |
| --- | --- |
| Flash once within 3 seconds | GSM initialization |
| Chang Liang | GSM communication is normal |
| Not lit up | GSM sleep/shutdown |

Blue light - GPS indicator light

| The status of the light | meaning |
| --- | --- |
| Flash once within 3 seconds | In satellite signal search |
| Chang Liang | GPS/BDS has been positioned |
| "Not bright" | GPS/BDS dormant |

### 1.4.3 Working logic

After the equipment is installed, and the yellow and blue lights are constantly on, the equipment enters its normal working state.

In the stationary and flameout state: the device sends one piece of data every 5 minutes.

Motion ignition status: The device sends one data point every 20 seconds.

- Features

- Multiple functions such as vibration alarm, overspeed alarm, and removal alarm;

- Support electronic fencing function, where users can define virtual electronic fences through the platform/APP. When devices enter or exit the fences, they can be recognized by the platform and an alarm will be issued;

- The terminal adopts an industrial-grade high-stability GPRS module from a well-known manufacturer, with a built-in high-sensitivity communication antenna, supporting TCP/IP data transmission and domain name/IP address connection to servers;

- Equipped with a large-capacity storage chip, it supports data storage in offline mode and the supplementary transmission of blind-spot data. When the vehicle is in an area with weak wireless signals or severe interference, the onboard device will temporarily store the vehicle's operational data in FLASH. Once the wireless signal returns to normal, it can transmit these data to ensure no data is lost.;

- Equipped with a built-in 3-axis accelerometer and integrated with precise acceleration algorithms, it can obtain the current vehicle posture and other vehicle conditions in real-time;

- High-sensitivity GPS/BDS dual-satellite positioning module, equipped with an anti-interference ceramic antenna, ensures more stable satellite signal search, and supports AGPS for rapid positioning tracking and synchronous timing;

- Support online remote upgrade and remote configuration of product parameters.

- Product Parameters

- 3.1 Hardware specification parameters

|  | Function Name | have | none | Project function description |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Electrical Characteristics | power supply method | ● |  | battery-powered |  |  |
|  | Operating voltage range | ● |  | DC 9V - 32V |  |  |
|  | operating current | ● |  | 12V/65mA |  |  |
|  | quiescent current | ● |  | 12V / average 10mA |  |  |
|  | Built-in battery capacity | ● |  | 50 mAh (3.7V polymer battery) |  |  |
| Environmental characteristics | Operating temperature range | ● |  | -20℃ - 75℃ |  |  |
|  | Storage temperature range | ● |  | -30℃ - 80℃ |  |  |
|  | Operating humidity range | ● |  | 10%-85% RH non-condensing |  |  |
| Communication characteristics | SIM card | ● |  | Nano SIM card |  |  |
|  | Communication frequency band | ● |  | LTE/4G | LTE-FDD:B1/B2/B3/B4/B5/B7/B8/B12/B13/ B18/B19/B20/ B25/B26/B28/B66/ LTE-TDD :B34/B38/B39/B40/B41 |
|  |  |  |  | GSM/2G | 850/900/1800/1900MHz |
|  | Communication antenna | ● |  | built-in antenna |  |  |
|  | Antenna specifications | ● |  | FPC Antenna |  |  |
| GPS/BD positioning characteristics | positioning method | ● |  | BeiDou + GPS + LBS |  |  |
|  | Cold start time | ● |  | Average 32 seconds |  |  |
|  | Hot start time | ● |  | On average, 1 second |  |  |
|  | Tracking sensitivity | ● |  | -162 dBm |  |  |
|  | positioning antenna | ● |  | built-in antenna |  |  |
|  | Antenna Specifications | ● |  | 25mm * 25mm * 2mm |  |  |
|  | GPS frequency band | ● |  | L1: 1575.42±1.023MHz |  |  |
|  | BeiDou frequency band | ● |  | B1: 1561.098±2.046MHz |  |  |
|  | Number of satellite channels | ● |  | 32 |  |  |
|  | positioning accuracy | ● |  | <10m（1σ） |  |  |
|  | Timing accuracy | ● |  | <30ns（1σ） |  |  |
|  | speed measurement accuracy | ● |  | <0.1m/s（1σ） |  |  |
|  | maximum acceleration | ● |  | 4g |  |  |
|  | maximum speed | ● |  | 515m/s |  |  |
|  | maximum height | ● |  | 18000m |  |  |
| external interface | Charging interface TYPE-A | ● |  | DC: 5V/2A; 9V/2A |  |  |
|  | Charging interface TYPE-C | ● |  | DC: 5V/2A; 9V/2A |  |  |
|  | SOS | ● |  | Support SOS button |  |  |
| Specifications | Host dimensions (length, width, height) | ● |  | 85mm*49.2mm*26mm |  |  |
|  | Shell material | ● |  | ABS+PC |  |  |
|  | host weight | ● |  | 41 grams |  |  |