Basic introduction to AK waterproof wiring products

- 1.1 Product Introduction

AK-GT02XSA is a vehicle positioning terminal meticulously crafted for the Internet of Vehicles. It integrates GSM wireless communication technology and GPS/BDS satellite navigation and positioning technology. The terminal adopts an industrial-grade, highly integrated, fully built-in antenna design. The device is equipped with a 3-axis sensor and intelligent power-saving wake-up function. It features DC detection, ACC detection, vibration alarm, power failure alarm, overspeed alarm, and illegal start alarm. When paired with a global positioning service platform, it enables real-time acquisition and tracking of vehicle data.

## 1.2 Application areas

Insurance industry, corporate fleet industry, automobile manufacturers/4S stores, individual users, electric new energy sector, passenger vehicles, taxi services, rental vehicles, etc.

## 1.3 Product images

- 1.4 Introduction to product usage

### 1.4.1 SIM card installation method

The device uses a standard SIM card. The card slot is of the self-ejecting type, with the chip side facing the antenna direction, and the card notch inserted into the slot with the notch facing inward.

### 1.4.2 Indicator light description

Yellow light - GSM indicator light

| The status of the light | meaning |
| --- | --- |
| Flash once within 2 seconds | GSM initialization |
| Chang Liang | GSM communication is normal |
| not bright | GSM sleep/shutdown |

Blue light - GPS indicator light

| The status of the light | meaning |
| --- | --- |
| Flash once within 2 seconds | In satellite signal search |
| Chang Liang | Located |
| not bright | dormancy |

1.4.3 Working logic

After the equipment is installed, and the yellow and blue lights are constantly on, the equipment enters its normal working state.

Working logic: When ACC is on, the device does not sleep and uploads a piece of data every 20 seconds. When ACC is off, the device uploads data after being idle for 10 minutes

"Dormant, after entering dormancy, the device only transmits heartbeat data. During the period of inactivity, if vibration is detected, the device will reset the count."

"10 minutes of sleep.".

Vibration alarm: After being armed, the vibration device will trigger a vibration alarm. Once the vibration alarm is triggered, it is necessary to remain stationary for 5 minutes before

The alarm can be triggered again.

Displacement alarm: When the device is armed and in a positioning state, set a displacement alarm distance. If the device travels beyond this line segment distance, an alarm will be triggered

A displacement alarm will be reported, and the device will automatically disarm.

Power-off alarm: After the device is plugged in, it will detect the power supply. If the power supply is disconnected, the device will trigger an alarm 3 seconds after the disconnection

Trigger a power-off alarm and upload it to the server.

Illegal start alarm: After activating the illegal start alarm, when the device is armed and ignited in the disarmed state, the device will go

Report illegal start alarm. (ACC needs to be properly connected)

- Features

- Supports multiple alarm functions such as vibration alarm, power failure alarm, overspeed alarm, and illegal start alarm;

- Support the electronic fence function, where users can define a virtual electronic fence through the platform/APP. When the device enters or exits the fence, it can be recognized by the platform and an alarm will be issued;

- When an alarm occurs, an alert message or call can be sent to the monitoring number, and the alarm can be uploaded to the platform simultaneously;

- The terminal adopts an industrial-grade high-stability GPRS module, equipped with a built-in GSM high-sensitivity antenna, supports TCP/IP data transmission, and supports connecting to servers via domain name/IP address;

- Equipped with a high-capacity storage chip, it supports data storage in offline mode and the supplementary transmission of blind-spot data. When the vehicle is in an area with weak wireless signals or severe interference, the onboard device will temporarily store the vehicle's operational data in FLASH. Once the wireless signal returns to normal, it can transmit these data, ensuring no data is lost.;

- Equipped with a built-in 3-axis accelerometer and integrated with precise acceleration algorithms, it can obtain real-time vehicle posture and other vehicle condition assessments;

- High-sensitivity GPS/BDS dual-satellite positioning module, equipped with an anti-interference ceramic antenna, ensures more stable satellite signal search. It supports AGPS for rapid positioning tracking and synchronous timing;

- In sleep mode, automatic reporting of messages such as abnormal vibration alarms, vehicle start-up, shutdown, and sleep;

- Low-power energy-saving mode, accurately and reliably determining the ignition and flameout status of the car, and intelligent sleep and wake-up mechanisms can reduce the average power consumption of the entire system.

- Product Parameters

- 3.1 Hardware specification parameters

| project | Function Name | have | none | Functional Description |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Electrical Characteristics | power supply method | ● |  | Powered by battery |  |  |
|  | Operating voltage range | ● |  | DC 9V - 95V |  |  |
|  | operating current | ● |  | 12V / average 40mA |  |  |
|  | quiescent current | ● |  | 12V/less than 10mA |  |  |
|  | Built-in battery capacity | ● |  | 60 mAh (3.7V polymer battery) optional |  |  |
| Environmental characteristics | Operating temperature range | ● |  | -20℃ - 75℃ |  |  |
|  | Storage temperature range | ● |  | -30℃ - 80℃ |  |  |
|  | Operating humidity range | ● |  | 10%-85% RH non-condensing |  |  |
| Communication characteristics | Communication module brand/chip model | ● |  | SIMCOM 7670SA |  |  |
|  | Communication frequency band | ● ● |  | LTE/4G | LTE-FDD:B1/B2/B3/B4/B5/B7/B8/B28/B66 |  |
|  |  |  |  | GSM/2G | 850/900/1800/1900MHz |  |
|  | SIM card | ● |  | Standard SIM card |  |  |
|  | Communication antenna | ● |  | built-in antenna |  |  |
| GPS/BD positioning characteristics | Positioning module brand/chip model | ● |  | Zhongke Micro AT6558D |  |  |
|  | positioning method | ● |  | BeiDou + GPS |  |  |
|  | Cold start time | ● |  | Average 32 seconds |  |  |
|  | Hot start time | ● |  | Average 1 second |  |  |
|  | Tracking sensitivity | ● |  | -162 dBm |  |  |
|  | positioning antenna | ● |  | built-in antenna |  |  |
|  | Antenna Specifications | ● |  | 25mm * 25mm * 4mm |  |  |
|  | GPS frequency band | ● |  | L1: 1575.42±1.023MHz |  |  |
|  | BeiDou frequency band | ● |  | B1: 1561.098±2.046MHz |  |  |
|  | Number of satellite channels | ● |  | 32 |  |  |
|  | positioning accuracy | ● |  | <10m（1σ） |  |  |
|  | Timing accuracy | ● |  | <30ns（1σ） |  |  |
|  | speed measurement accuracy | ● |  | <0.1m/s（1σ） |  |  |
|  | maximum acceleration | ● |  | 4g |  |  |
|  | maximum speed | ● |  | 515m/s |  |  |
|  | maximum height | ● |  | 18000m |  |  |
| external interface | Cut off fuel and electricity | ● |  | Optional features |  |  |
|  | ACC detection input | ● |  |  |  |  |
| Specifications | Host dimensions (length, width, height) | ● |  | 90.6mm * 34mm * 20mm |  |  |
|  | Shell material | ● |  | ABS plastic |  |  |
|  | IP protection rating | ● |  | IP67 |  |  |
|  | host weight | ● |  | 68 grams |  |  |