YG119 Hunting Dog GPS Tracker

User Manual

Description

Hunting dog tracker is a rechargeable GPS tracker with long standby battery. Only need insert SIM card, it will upload GPS location based on movement or preset frequency automatically.

Production Features

- Real time tracking via tracking APP

- Geo-fence/Low battery alarm

- Real time tracking/power saving/Deeply sleep mode.

- Find your pet via buzzer sound

- Electric shock training for dogs

- Magnet charging point

- Waterproof IP68

Specs.

| GSM Specification |  |
| --- | --- |
| 4G LTE bands | 4G: LTE-FDD B1/B2/B3/B4/B5/B7/B8/B28/B66 LTE-TDD B34/B38/B39/B40/B41 2G: GSM850/GSM900/1800/GSM1900 |
| GPRS | Class 12, TCP/IP |
| Memory | 32 + 32 Mb |
| Phase error | RMSPE<5,PPE<20 |
| Max output | GSM850/GSM900:33±3dBm ,GSM1800/GSM1900:30±3dBm |
| Max frequency error | ±0.1ppm |
| Receive sensitivity | Class II RBER2%(-102dBm) |
| GPS Specification |  |
| Chipset | SIMCom LTE Cat 1 A7670SA + UC6228CI (GPS) |
| Frequency | GPS L1, 1575.42 MHz C/A code |
| Channel | 66 |
| Location accuracy | <5meters |
| Tracking sensitivity | -165dBm |
| Acquisition sensitivity | -148dBm |
| TTFF (open sky) | Avg. hot start ≤1 sec |
|  | Avg. cold start≤40 sec |
| Others |  |
| Antenna | Built-in quad band GSM antenna, GPS ceramic antenna |
| Charging voltage | 5VDC |
| Working current | 2mA~78mA |
| Electric shock current | Instantaneous voltage 6800V |
| Battery | rechargeable 4500 lithium battery |
| Standby time | Everyday move 1 hour activities can standby 14 days |
| Operating temperature | -20℃~+60℃ |
| Unit Size | 78*40*44MM |
| Waterproof | IP68 |

- Power on button

- Speaker/Microphone

- Electronic shock Point

- Charging Point

Start Using

1. Insert a SIM card with 4g data activated.

2. Press the backside circle button 3 seconds to power on, and beep one time.
Press button 6 seconds to power off, and beep one time.

3. Put back back shell and tighten the screws for 100% waterproof

| Light | Signal | Normal | Abnormal |
| --- | --- | --- | --- |
| Green | GSM | Flashing | Constant on |
| Blue | GPS | Flashing | Constant on |
| Red | Power | Constant on: charging | Turn off: charged full |
| Device will power off automatically during charging. |  |  |  |

SMS Command List

| SMS Function | SMS Command format |  | SMS example |  |
| --- | --- | --- | --- | --- |
| - Please don’t type “+” “space”, password 123456 - Preset Center Number first before testing SMS or Phone Call alarm mode. - Each SMS must reply“OK” represent command successfully. |  |  |  |  |
| Mode 0 |  | mod123456+space+0 |  | Real time mode consume battery fast |
| Mode 1 |  | mod123456+space+1 |  | Vibrate mode based on movement |
| Mode 2 |  | mod123456+space+2 |  | Deeply sleep mode wakeup 3 minutes at every 2hrs |
| Get location now |  | DM123456 |  | Get current location update 1 time |
| Find Pet via buzzer |  | Find123456 |  | Buzzer sound 10seconds |
| Set IP Port |  | adminip+password+space+IP Address+space+port number |  | adminip123456 www.gps2828.com 7018 |
| Set APN |  | apn123456+space+LocalApn+space+ApnName+space+ApnPassword |  | apn123456 internet |
| Set Time Interval |  | ITV123456+space+time interval(s) |  | If 60s , send SMS: ITV123456 60 |
| Set admin number |  | Admin123456+space+PhoneNumber |  | You can set max 5 numbers Admin123456 13702581234 13702581235 13702581236 |
| Set low battery alarm |  | lowbat123456+space+on |  | lowbat123456 on |
| Cancel low battery |  | lowbat123456+space+off |  | lowbat123456 off |
| Set SMS time zone |  | Time+space+zone123456+space+X |  | If+3,send sms time zone123456 3 |
| Check device status |  | check123456 |  | check GPS GSM status |
| Activate voice listen |  | Monitor123456 |  |  |
| Cancel voice isten |  | Tracker123456 |  |  |
| Set alarm mode |  | KC123456+space+0 |  | Cancel alarm |
|  |  | KC123456+space+1 |  | APP alarm (default) |
|  |  | KC123456+space+2 |  | APP+SMS alarm |
| Set factory mode |  | begin123456 |  | It will erase all setting |

Real time Tracking Software

Web:    www.gps16888.com

Android & IOS APP : toptrace

Default to use IMEI number login, password as 123456.