Introduction to K6 product specifications

Product images

Product Overview

- ● Product Name: K6-4G Bicycle Locator

● Application scenarios: bicycles, mountain bikes, and other cycling series

● Core functions: real-time positioning, historical track playback, electronic fence, wifi positioning, overspeed alarm, vibration alarm, low battery alarm, etc

- ● Product Highlights: High-precision positioning (5 meters), four-mode positioning (GPS + Beidou + LBS + WIFI), long battery life, strobe warning lights, and bracket installation

One-click car locator (with buzzer)

Technical Specifications

| project | Function Name | have | none | Functional Description |
| --- | --- | --- | --- | --- |
| Power Characteristics | power supply method | ● |  | battery-powered |
|  | Operating voltage range | ● |  | DC 3.45V - 4.35V |
|  | operating current | ● |  | 3.7V/45mA (GPS upload for one minute) |
|  | quiescent current | ● |  | 3.7V/4mA |
|  | Built-in battery capacity | ● |  | 1500 mAh (3.8V polymer battery) |
| Environmental characteristics | Operating temperature range | ● |  | -20℃ - 70℃ |
|  | Storage temperature range | ● |  | -40℃ - 85℃ |
|  | Operating humidity range | ● |  | 5%-95% |
| Communication characteristics | Communication module brand/chip model | ● |  | ASR1603S |
|  | communication frequency band | ● |  | GSM：900/1800MHZ FDD-LTE:B1/3/5/8 TDD-LTE:B38/39/40/41 |
|  | SIM card | ● |  | Nano SIM card |
|  | communication antenna | ● |  | Built-in FPC antenna |
| GPS/BD positioning features | Cold start time | ● |  | On average, it takes 45 seconds |
|  | Hot start time | ● |  | On average, 1 second |
|  | Sensitivity (tracking) | ● |  | -165dbm |
|  | positioning antenna | ● |  | Built-in antenna FPC antenna |
| Hardware function | USB | ● |  | Type-C, charging, activating battery |
|  | LED | ● |  | Strobe warning light |
|  | buzzer | ● |  | One-click car locating |
|  | Gsensor | ● |  | built-in |
|  | GPS | ● |  | Support global GPS and Beidou |
|  | WiFi | ● |  | WiFi positioning |
| Outline specifications | Host dimensions (length, width, height) | ● |  | 73mm × 60.5 mm × 29 mm |
|  | Shell material | ● |  | ABS plastic |
|  | IP protection rating | ● |  | IP65 |
|  | bicycle stand | ● |  | ABS plastic + silicone gasket |
|  | host weight | ● |  | ~85 grams |

Software features

Real-time tracking: View real-time locations through the APP/Web terminal, with update frequencies adjustable in multiple modes (10 seconds to 3 minutes).

Electronic fence: Customize a security zone, and trigger an APP push alert when entering or exiting.

Historical track: Stores and replays movement paths within 180 days, supports exporting in KML/GPX formats.

Multi-account management: Supports family/team sharing of device permissions.

OTA upgrade: Remote wireless firmware upgrade, with continuous optimization of functions.