Product images

Detailed specifications:

- Precise positioning, with an accuracy of 1.5 meters (the product supports Beidou III).

- GPS + Beidou + LBS + WIFI four-mode positioning.

- Practical functions: real-time positioning, latent positioning, vibration wake-up, and step counting positioning.

- Dismantling alarm, vibration alarm, displacement alarm, low battery alarm, fence exit alarm, overspeed alarm, etc.

- Small in size, easy to conceal, with strong magnetic attraction (optional).

Application fields:

Leased vehicles, credit vehicles, passenger vehicles, taxis, freight vehicles, cattle and sheep positioning management, valuables management, etc

Device parameters:

| project | Function Name | have | none | Functional Description |
| --- | --- | --- | --- | --- |
| Electrical Characteristics | power supply method | ● |  | battery-powered |
|  | Operating voltage range | ● |  | DC 3.45V - 4.35V |
|  | operating current | ● |  | 22mA |
|  | Standby current | ● |  | About 30uA |
|  | Online standby current | ● |  | 4.5mA |
|  | Built-in battery capacity | ● |  | 10000mAh (3.8V polymer battery) |
| Environmental characteristics | Operating temperature range | ● |  | -20℃ - 70℃ |
|  | Storage temperature range | ● |  | -30℃ - 80℃ |
|  | Operating humidity range | ● |  | 5%-95% |
| Communication characteristics | Communication module brand/chip model | ● |  | ML307A-DCLN-China Mobile IoT |
|  | communication frequency band | ● |  | TDD:B34/38/39/40/41,FDD:B1/3/5/8 |
|  | SIM card | ● |  | Micro SIM card |
|  | communication antenna | ● |  | Built-in FPC antenna |
| GPS/BD positioning features | Positioning module brand/chip model | ● |  | BGI GPS Beidou chip (supporting Beidou-3) |
|  | Cold start time | ● |  | Average 32 seconds |
|  | Hot start time | ● |  | Average: 1 second |
|  | Sensitivity (tracking) | ● |  | -162dBm |
|  | positioning antenna | ● |  | Built-in active ceramic GPS antenna |
| external interface | Loudspeaker output interface |  | ● |  |
|  | I/O interface |  | ● |  |
|  | ADC detection input interface |  | ● |  |
|  | RS485 interface |  | ● |  |
|  | RS232 interface |  | ● |  |
|  | serial port | ● |  | 3.3V serial port channel 1 |
|  | CAN |  | ● |  |
|  | K-line |  | ● |  |
| Outline specification | Host dimensions (length, width, height) | ● |  | 75mm × 57 mm × 31.5 mm |
|  | Shell material | ● |  | ABS plastic |
|  | IP protection rating | ● |  | IP65 |
|  | strong magnet | ● |  | optional |
|  | collar | ● |  | optional |
|  | host weight | ● |  | About 199 grams |