| NY Solar Product Specifications |  |  |
| --- | --- | --- |
| project | description |  |
| product image |  |  |
| Product Name | NY high-capacity positioning | Supports TDD/FDD/GSM mobile network access services, GPS positioning, Beidou positioning, and WIFI positioning |
| structural characteristics | Overall dimensions | 122*84*48.5mm |
|  | Unit Weight | 400g/551g |
| data services | FDD/TDD | Upstream rate of 5Mbps, downstream rate of 10Mbps |
|  | GSM | The upstream rate is 48kbps, and the downstream rate is 54kbps |
| Communication network frequency band | Full Netcom | China Mobile, China Unicom, China Telecom |
|  | FDD frequency band | B1,3,5,8 |
|  | TDD frequency band | B38,39,40,41 |
|  | GSM frequency band | 900/1800MhZ |
| GPS frequency band | Beidou | 1561.098MHz |
|  | GPS | 1575.42MHz |
| GPS sensitivity | Acquisition | -148dBm(cold)/-163dBm(hot) |
|  | Tracking (tracking) | -165dBM |
|  | power consumption | Acq:37mW,Tracking:27mW |
| WIFI function | support | Obtain MAC address, save power |
| Power Characteristics | Working Voltage | 3.5-4.2V DC |
|  | voltage fluctuation | 3.3-4.5V DC |
|  | operating current | Sleep 0.3mA (maintaining clock) Standby 3.3mA (3-minute heartbeat) Positioning 60mA (LTE+GPS) |
|  | Built-in battery | 10000mAh/20000mAh |
| Environmental characteristics | Operating Temperature | -20℃～+70℃ |
|  | Ultimate operating temperature | -30℃～+75℃ |
|  | Storage temperature | -40℃～+85℃ |
|  | relative humidity | 5%～95% |
| Main hardware functions | Gsensor | built-in |
|  | GPS | Support global GPS and Beidou |
|  | solar energy |  |
|  | LED indicator light | GPS (blue), GSM (green), Power (red) |
|  | SIM interface | Nano, push-push side insertion, compatible with 5*6 ESIM |
|  | USB | Type-C, charging + downloading, testing, activating battery |
|  | Simulated voice | Built-in dual microphones (interruption detection + recording) |
|  | watchdog | Support, built-in chip |
|  | antenna | GPS: Passive dual-mode ceramic 18*18; LTE: FPC (with spring clips on the mainboard) WIFI: FPC (mainboard with spring clip) |
| Main software functions | Smart wake-up/sleep | Support, timing or Gsensor |
|  | positioning | Support, scheduled positioning, intelligent positioning, super power-saving positioning |
|  | WIFI positioning | support |
|  | Anti-theft recording | support |
|  | FOTA | support |