| Q8 Elderly Locator Product Specification |  |  |
| --- | --- | --- |
|  | Elderly care tracker | Supports TDD/FDD/GSM mobile network access services, GPS positioning, Beidou positioning, and WIFI positioning |
|  | Overall dimensions | 68*38*20 mm |
|  | button | Option 1: Power button (compatible with SOS function) Option 2: Customized buttons for BTB connector: including power button, LED indicator, on/off switch, SOS, and others |
|  | FDD/TDD | The upstream rate is 5Mbps, and the downstream rate is 10Mbps |
|  | GSM | The upstream rate is 48kbps, and the downstream rate is 54kbps |
|  | B1,3,5,8 |  |
|  | B38,39,40,41 |  |
|  | 900/1800MhZ |  |
|  | Beidou | 1561.098MHz |
|  | GPS | 1575.42MHz |
|  | Acquisition | -148dBm(cold)/-163dBm(hot) |
|  | Tracking (tracking) | -165dBM |
|  | power consumption | Acq:37mW,Tracking:27mW |
|  | support | Obtain MAC address, save power |
|  | support | Optional, Bluetooth anti-lost feature |
|  | Working Voltage | 3.5-4.2V DC |
|  | voltage fluctuation | 3.3-4.5V DC |
|  | operating current | Sleep 0.3mA (maintain clock) Standby 3.3mA (3-minute heartbeat), positioning 60mA (LTE+GPS) |
|  | Built-in battery | 1500mAh high-voltage high-capacity polymer lithium battery |
|  | Operating Temperature | -20℃~+70℃ |
|  | Ultimate operating temperature | -30℃~+75℃ |
|  | Storage temperature | -40℃~+85℃ |
|  | relative humidity | 5%~95% |
|  | Gsensor | built-in |
|  | GPS | Support global GPS and Beidou |
|  | LED indicator light | GPS (blue), GSM (green), Power (red) |
|  | SIM interface | Nano, push-push side insertion, compatible with 2*2 eSIM |
|  | USB | Option 1: Type-C, charging + downloading, testing, activating battery Option 2: Magnetic attraction function, utilizing the 10-pin connector of the motherboard |
|  | Simulated voice | Built-in single microphone (interrupt detection + recording), with options for surface mount or wire bond depending on appearance |
|  | horn | 8Ω, 1.2W or 4Ω, 2.4W |
|  | watchdog | Support, chip comes with it |
|  | antenna | GPS: FPC or LDS (with spring clips on the mainboard) LTE: FPC or LDS (mainboard with spring clip) WIFI: the mainboard comes with a ceramic patch antenna |
|  | Smart wake-up/sleep | Support, timing or Gsensor |
|  | positioning | Support, scheduled positioning, intelligent positioning, and super power-saving positioning |
|  | WIFI positioning | support |
|  | Bluetooth anti-lost | support |
|  | Anti-theft recording | support |
|  | FOTA | support |